LAB 6: DYNAMIC ROUTING CONFIGURATION:
RIP, EIGRP, OSPF, BGP

# OBJECTIVES

a) To study and understand the working principles of dynamic routing protocols: RIP, EIGRP, OSPF, and BGP.

b) To configure these routing protocols on Cisco routers using Packet Tracer to enable automatic route sharing between networks.

# HARDWARE AND SOFTWARE REQUIREMENTS

1. Cisco Packet Tracer (Version 6.2 or higher)

2. Windows PC/Laptop

# THEORY

1. RIP (Routing Information Protocol)

RIP is a distance-vector routing protocol that uses hop count to find the best path. It sends the entire routing table to neighbors every 30 seconds. The maximum hop count is 15, and it is suitable for small networks due to slow convergence.

2. EIGRP (Enhanced Interior Gateway Routing Protocol)

EIGRP is a Cisco advanced routing protocol that uses the DUAL algorithm to find the best path. It sends only partial updates when changes occur, saving bandwidth. EIGRP is used in Cisco networks that need fast convergence.

3. OSPF (Open Shortest Path First)

OSPF is a link-state routing protocol that uses the Dijkstra algorithm to calculate the shortest path. Routers exchange LSAs to build a network map. OSPF supports network division into areas, with Area 0 as the backbone, making it suitable for large networks.

4. BGP (Border Gateway Protocol)

BGP is a path-vector protocol used to exchange routing information between different autonomous systems. It makes decisions based on network policies and is the main protocol for Internet routing between ISPs.

# NETWORK TOPOLOGY

A network topology was created using Router0 (ANSHU) and Router1 (ANSHU0) connected via a serial link. Switch0 connects PC0 and PC1 to Router0 (Network 192.168.1.0/24). Switch1 connects PC2 and PC3 to Router1 (Network 192.168.2.0/24). The serial link uses the 10.0.0.0/8 network.

# IP CONFIGURATION TABLE

| Device | Interface | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| Router0 | Gig0/1 | 192.168.1.1 | 255.255.255.0 | N/A |
| Router0 | Gig0/0 | 10.0.0.1 | 255.0.0.0 | N/A |
| Router1 | Gig0/1 | 192.168.2.1 | 255.255.255.0 | N/A |
| Router1 | Gig0/0 | 10.0.0.2 | 255.0.0.0 | N/A |
| PC0 | FastEthernet0 | 192.168.1.2 | 255.255.255.0 | 192.168.1.1 |
| PC1 | FastEthernet0 | 192.168.1.3 | 255.255.255.0 | 192.168.1.1 |
| PC2 | FastEthernet0 | 192.168.2.2 | 255.255.255.0 | 192.168.2.1 |
| PC3 | FastEthernet0 | 192.168.2.3 | 255.255.255.0 | 192.168.2.1 |

# ROUTING PROTOCOL CONFIGURATION COMMANDS

1. RIP Configuration Commands

Router0:

Router(config)# router rip

Router(config-router)# version 2

Router(config-router)# network 192.168.1.0

Router(config-router)# network 10.0.0.0

Router(config-router)# no auto-summary

Router1:

Router(config)# router rip

Router(config-router)# version 2

Router(config-router)# network 192.168.2.0

Router(config-router)# network 10.0.0.0

Router(config-router)# no auto-summary

2. EIGRP Configuration Commands

Router0:

Router(config)# router eigrp 100

Router(config-router)# network 192.168.1.0 0.0.0.255

Router(config-router)# network 10.0.0.0 0.0.0.255

Router(config-router)# no auto-summary

Router1:

Router(config)# router eigrp 100

Router(config-router)# network 192.168.2.0 0.0.0.255

Router(config-router)# network 10.0.0.0 0.0.0.255

Router(config-router)# no auto-summary

3. OSPF Configuration Commands

Router0:

Router(config)# router ospf 1

Router(config-router)# network 192.168.1.0 0.0.0.255 area 0

Router(config-router)# network 10.0.0.0 0.0.0.255 area 0

Router1:

Router(config)# router ospf 1

Router(config-router)# network 192.168.2.0 0.0.0.255 area 0

Router(config-router)# network 10.0.0.0 0.0.0.255 area 0

4. BGP Configuration Commands

Router0 (AS 65001):

Router(config)# router bgp 65001

Router(config-router)# neighbor 10.0.0.2 remote-as 65002

Router(config-router)# network 192.168.1.0 mask 255.255.255.0

Router1 (AS 65002):

Router(config)# router bgp 65002

Router(config-router)# neighbor 10.0.0.1 remote-as 65001

Router(config-router)# network 192.168.2.0 mask 255.255.255.0

# OBSERVATION / PING TEST RESULTS

| Protocol | Source Device | Destination Device | Packets Sent | Result |
| --- | --- | --- | --- | --- |
| RIP | PC0 (192.168.1.2) | PC2 (192.168.2.2) | 4 | Successful |
| EIGRP | PC0 (192.168.1.2) | PC2 (192.168.2.2) | 4 | Successful |
| OSPF | PC0 (192.168.1.2) | PC2 (192.168.2.2) | 4 | Successful |
| BGP | PC0 (192.168.1.2) | PC2 (192.168.2.2) | 4 | Successful |

# RESULT

The network was successfully configured using different dynamic routing protocols. Ping tests from PC0 (192.168.1.2) to PC2 (192.168.2.2) were successful for all protocols (RIP, EIGRP, OSPF, and BGP). The routing tables were automatically updated with learned routes, confirming proper configuration.

# DISCUSSION AND CONCLUSION

In this lab, we successfully configured and tested four dynamic routing protocols. RIP showed simple configuration but has limitations for larger networks. EIGRP demonstrated faster convergence with efficient updates. OSPF provided scalability through area design. BGP enabled routing between autonomous systems using policies.

The ping tests confirmed that all protocols worked correctly, and routes were learned automatically. This hands-on experience helped us understand how to choose and configure the right routing protocol based on network requirements.