Design a small campus with 2 buildings connected by routers running OSPF. Create VLANs for Students and Staff at each building. Configure DHCP servers. Setup a DNS server. Practice subnetting to allocate IP addresses efficiently. Test with ping and traceroute.

Objectives

- To design and implement a small campus network with two buildings connected by routers running OSPF dynamic routing protocol.

- To configure VLANs for Students and Staff in each building, set up DHCP servers for automatic IP assignment, and configure a DNS server for name resolution.

- To practice subnetting for efficient IP address allocation and verify full network connectivity using ping and traceroute commands.

Theory

1. VLAN (Virtual Local Area Network)

Definition: A VLAN is a logical grouping of devices on a network, separated at Layer 2, regardless of their physical location.

Operation: Switches tag frames with a VLAN ID using IEEE 802.1Q. Access ports carry traffic for a single VLAN; trunk ports carry multiple VLANs simultaneously between switches and routers.

Application: Used to separate Students and Staff into different broadcast domains within each building for security and traffic management.

2. OSPF (Open Shortest Path First)

Definition: OSPF is a link-state dynamic routing protocol that automatically discovers and shares routing information between routers.

Operation: Routers exchange Link State Advertisements (LSAs) to build a complete map of the network. Each router calculates the shortest path using Dijkstra's algorithm. Neighbor state FULL means routing tables have been fully exchanged.

Application: Used to allow Router-A and Router-B to automatically learn each other's networks without static routes.

3. DHCP (Dynamic Host Configuration Protocol)

Definition: DHCP automatically assigns IP addresses, subnet masks, default gateways, and DNS server IPs to devices on a network.

Operation: A client broadcasts a DHCP Discover message. The server responds with an Offer, the client sends a Request, and the server sends an Acknowledgment (DORA process). The ip helper-address command on the router forwards DHCP broadcasts across VLAN boundaries.

Application: Used so all 6 PCs across 4 VLANs receive correct IP configuration automatically without manual setup.

4. DNS (Domain Name System)

Definition: DNS translates human-readable domain names into IP addresses.

Operation: A client sends a query for a domain name (e.g., campus.local) to the configured DNS server. The server looks up its A records and returns the corresponding IP address.

Application: Configured so devices can reach network resources using names like campus.local and buildingB.local instead of IP addresses.

5. Subnetting

Definition: Subnetting divides a larger IP network into smaller, more efficient sub-networks.

Operation: By adjusting the subnet mask, available host bits are used to create separate address spaces. A /26 mask provides 62 usable hosts; /27 provides 30; /30 provides 2.

Application: Applied to allocate appropriately sized subnets — larger /26 for Student VLANs and smaller /27 for Staff VLANs.

Network Topology

The campus network consists of two buildings, each containing one ISR4331 router, one 2960-24TT switch, one server, two student PCs, and one staff PC — 10 devices in total. Router-A and Router-B are connected via a crossover WAN link (10.0.0.0/30). Within each building, the router connects to the switch via a trunk link (Gig0/1) carrying both VLANs. Server-DHCP-DNS in Building A serves both DHCP and DNS for the entire network. Server-DHCP-B in Building B handles DHCP for its local VLANs.

Figure 1: Campus Network Topology in Cisco Packet Tracer (all green links = active)

IP Addressing Scheme

| Device | Interface | IP Address | Subnet Mask | Gateway | VLAN / Role |
| --- | --- | --- | --- | --- | --- |
| Router-A | Gig0/0/0 | 10.0.0.1 | 255.255.255.252 | — | WAN |
| Router-A | Gig0/0/1.10 | 192.168.10.1 | 255.255.255.192 | — | VLAN 10 GW |
| Router-A | Gig0/0/1.20 | 192.168.20.1 | 255.255.255.224 | — | VLAN 20 GW |
| Router-B | Gig0/0/0 | 10.0.0.2 | 255.255.255.252 | — | WAN |
| Router-B | Gig0/0/1.30 | 192.168.30.1 | 255.255.255.192 | — | VLAN 30 GW |
| Router-B | Gig0/0/1.40 | 192.168.40.1 | 255.255.255.224 | — | VLAN 40 GW |
| Server-DHCP-DNS | Fa0 | 192.168.10.10 | 255.255.255.192 | 192.168.10.1 | DHCP+DNS |
| Server-DHCP-B | Fa0 | 192.168.30.10 | 255.255.255.192 | 192.168.30.1 | DHCP |
| PC-Student-A1 | Fa0 | 192.168.10.20 | 255.255.255.192 | 192.168.10.1 | VLAN 10 |
| PC-Student-A2 | Fa0 | 192.168.10.21 | 255.255.255.192 | 192.168.10.1 | VLAN 10 |
| PC-Staff-A1 | Fa0 | 192.168.20.21 | 255.255.255.224 | 192.168.20.1 | VLAN 20 |
| PC-Student-B1 | Fa0 | 192.168.30.20 | 255.255.255.192 | 192.168.30.1 | VLAN 30 |
| PC-Student-B2 | Fa0 | 192.168.30.21 | 255.255.255.192 | 192.168.30.1 | VLAN 30 |
| PC-Staff-B1 | Fa0 | 192.168.40.20 | 255.255.255.224 | 192.168.40.1 | VLAN 40 |

Note: PC IPs assigned via DHCP starting from .20. Servers and router interfaces use static IPs.

Configuration / Procedure

Step 1: Device Placement and Cabling

- Opened Cisco Packet Tracer and placed 2x ISR4331 routers, 2x 2960-24TT switches, 2x Server-PT, and 6x PC-PT devices on the canvas.

- Connected each switch to its router using Copper Straight-Through cable (Switch Gig0/1 to Router Gig0/0/1).

- Connected Router-A to Router-B using Copper Cross-Over cable (Gig0/0/0 on both).

- Connected all PCs and servers to their building's switch using Copper Straight-Through cables.

Step 2: VLAN Configuration — Switch A

Created VLAN 10 (Students-A) and VLAN 20 (Staff-A). Assigned Fa0/1-Fa0/3 to VLAN 10, Fa0/4 to VLAN 20, and set Gig0/1 as trunk.

Switch> enable Switch# configure terminal Switch(config)# vlan 10 Switch(config-vlan)# name Students-A Switch(config-vlan)# exit Switch(config)# vlan 20 Switch(config-vlan)# name Staff-A Switch(config-vlan)# exit Switch(config)# interface fastEthernet 0/1 Switch(config-if)# switchport mode access Switch(config-if)# switchport access vlan 10 ! Server-DHCP-DNS Switch(config)# interface fastEthernet 0/2 Switch(config-if)# switchport access vlan 10 ! PC-Student-A1 Switch(config)# interface fastEthernet 0/3 Switch(config-if)# switchport access vlan 10 ! PC-Student-A2 Switch(config)# interface fastEthernet 0/4 Switch(config-if)# switchport mode access Switch(config-if)# switchport access vlan 20 ! PC-Staff-A1 Switch(config)# interface gigabitEthernet 0/1 Switch(config-if)# switchport mode trunk Switch(config-if)# switchport trunk allowed vlan 10,20 Switch(config-if)# exit Switch# write memory

Step 3: VLAN Configuration — Switch B

Same procedure using VLAN 30 (Students-B) and VLAN 40 (Staff-B).

Switch(config)# vlan 30 Switch(config-vlan)# name Students-B Switch(config)# vlan 40 Switch(config-vlan)# name Staff-B Switch(config)# interface fastEthernet 0/1 Switch(config-if)# switchport access vlan 30 ! Server-DHCP-B Switch(config)# interface fastEthernet 0/2 Switch(config-if)# switchport access vlan 30 ! PC-Student-B1 Switch(config)# interface fastEthernet 0/3 Switch(config-if)# switchport access vlan 30 ! PC-Student-B2 Switch(config)# interface fastEthernet 0/4 Switch(config-if)# switchport access vlan 40 ! PC-Staff-B1 Switch(config)# interface gigabitEthernet 0/1 Switch(config-if)# switchport mode trunk Switch(config-if)# switchport trunk allowed vlan 30,40 Switch# write memory

Step 4: Router A Configuration (Sub-interfaces + WAN)

Configured Router-on-a-Stick — one physical interface split into sub-interfaces, one per VLAN using encapsulation dot1Q.

Router(config)# hostname Router-A Router-A(config)# interface GigabitEthernet0/0/0 Router-A(config-if)# ip address 10.0.0.1 255.255.255.252 Router-A(config-if)# no shutdown Router-A(config)# interface GigabitEthernet0/0/1 Router-A(config-if)# no shutdown Router-A(config)# interface GigabitEthernet0/0/1.10 Router-A(config-subif)# encapsulation dot1Q 10 Router-A(config-subif)# ip address 192.168.10.1 255.255.255.192 Router-A(config)# interface GigabitEthernet0/0/1.20 Router-A(config-subif)# encapsulation dot1Q 20 Router-A(config-subif)# ip address 192.168.20.1 255.255.255.224 Router-A(config)# end Router-A# write memory

Step 5: Router B Configuration (Sub-interfaces + WAN)

Same Router-on-a-Stick technique for Building B using VLAN 30 and VLAN 40.

Router(config)# hostname Router-B Router-B(config)# interface GigabitEthernet0/0/0 Router-B(config-if)# ip address 10.0.0.2 255.255.255.252 Router-B(config-if)# no shutdown Router-B(config)# interface GigabitEthernet0/0/1 Router-B(config-if)# no shutdown Router-B(config)# interface GigabitEthernet0/0/1.30 Router-B(config-subif)# encapsulation dot1Q 30 Router-B(config-subif)# ip address 192.168.30.1 255.255.255.192 Router-B(config)# interface GigabitEthernet0/0/1.40 Router-B(config-subif)# encapsulation dot1Q 40 Router-B(config-subif)# ip address 192.168.40.1 255.255.255.224 Router-B(config)# end Router-B# write memory

Step 6: OSPF Configuration

Enabled OSPF process 1 on both routers and announced all connected networks using wildcard masks.

! On Router-A: Router-A(config)# router ospf 1 Router-A(config-router)# network 10.0.0.0 0.0.0.3 area 0 Router-A(config-router)# network 192.168.10.0 0.0.0.63 area 0 Router-A(config-router)# network 192.168.20.0 0.0.0.31 area 0 Router-A# write memory ! On Router-B: Router-B(config)# router ospf 1 Router-B(config-router)# network 10.0.0.0 0.0.0.3 area 0 Router-B(config-router)# network 192.168.30.0 0.0.0.63 area 0 Router-B(config-router)# network 192.168.40.0 0.0.0.31 area 0 Router-B# write memory

Step 7: DHCP Server Configuration

Configured DHCP pools on both servers via Services > DHCP. Added ip helper-address on router sub-interfaces to relay DHCP requests across VLANs.

| Pool Name | Server | Start IP | Subnet Mask | Gateway | DNS Server |
| --- | --- | --- | --- | --- | --- |
| Students-A | Server-DHCP-DNS | 192.168.10.20 | 255.255.255.192 | 192.168.10.1 | 192.168.10.10 |
| Staff-A | Server-DHCP-DNS | 192.168.20.20 | 255.255.255.224 | 192.168.20.1 | 192.168.10.10 |
| Students-B | Server-DHCP-B | 192.168.30.20 | 255.255.255.192 | 192.168.30.1 | 192.168.10.10 |
| Staff-B | Server-DHCP-B | 192.168.40.20 | 255.255.255.224 | 192.168.40.1 | 192.168.10.10 |

! ip helper-address — forwards DHCP requests across VLANs: Router-A(config)# interface GigabitEthernet0/0/1.20 Router-A(config-subif)# ip helper-address 192.168.10.10 Router-B(config)# interface GigabitEthernet0/0/1.40 Router-B(config-subif)# ip helper-address 192.168.30.10

Step 8: DNS Server Configuration

Enabled DNS service on Server-DHCP-DNS (Services > DNS > Service ON) and added four A Records.

| Domain Name | Record Type | IP Address |
| --- | --- | --- |
| campus.local | A Record | 192.168.10.10 |
| buildingA.local | A Record | 192.168.10.1 |
| buildingB.local | A Record | 192.168.30.1 |
| server-b.local | A Record | 192.168.30.10 |

Result / Testing / Verification

OSPF Neighbor Verification — show ip ospf neighbor

The command show ip ospf neighbor was run on both routers to confirm OSPF adjacency. State FULL confirms complete routing table exchange.

Figure 2: Router-A — Neighbor 192.168.40.1, State: FULL/BDR, Address: 10.0.0.2

Figure 3: Router-B — Neighbor 192.168.20.1, State: FULL/DR, Address: 10.0.0.1

Ping Test — Building A to Building B (Cross-Building)

ping 192.168.30.20 was run from PC-Student-A1 to PC-Student-B1. 75% success — first packet timed out due to ARP resolution (normal behaviour). Remaining 3 packets received replies confirming cross-building connectivity.

Figure 4: ping 192.168.30.20 — Building A to Building B (75%, ARP first packet normal)

Ping Test — Inter-VLAN (Students to Staff, same building)

ping 192.168.20.21 was run from PC-Student-A1 (VLAN 10) to PC-Staff-A1 (VLAN 20). 100% success with 0% packet loss confirming inter-VLAN routing via Router-A sub-interfaces.

Figure 5: ping 192.168.20.21 — Inter-VLAN Students to Staff (100% success, 0% loss)

Traceroute — 3-Hop Path Verification

tracert 192.168.30.20 was run from PC-Student-A1 to confirm the path packets take. Output shows 3 hops: Router-A (192.168.10.1), Router-B WAN (10.0.0.2), and destination PC (192.168.30.20).

Figure 6: tracert 192.168.30.20 — 3-hop path: Router-A to Router-B to Destination

DNS Verification — nslookup campus.local

nslookup campus.local was run from PC-Student-A1. DNS server at 192.168.10.10 correctly resolved campus.local to 192.168.10.10 confirming DNS is operational.

Figure 7: nslookup campus.local — resolved to 192.168.10.10 by DNS server

DNS Ping — ping campus.local

ping campus.local was executed to confirm end-to-end DNS resolution. The PC resolved the name to 192.168.10.10 and received 4/4 replies with 0% packet loss.

Figure 8: ping campus.local — DNS resolved and 100% ping success

DNS Ping — ping buildingB.local (Cross-Building DNS)

ping buildingB.local verified that DNS resolution works across buildings. The name resolved to 192.168.30.1 and all 4 packets received replies.

Figure 9: ping buildingB.local — resolved to 192.168.30.1, 100% success

Discussion and Conclusion

In this project, a fully functional campus network was successfully designed and implemented in Cisco Packet Tracer. The network comprised two buildings, each with dedicated VLANs separating student and staff traffic. The Router-on-a-Stick technique was applied on both routers to enable inter-VLAN routing using 802.1Q sub-interfaces. OSPF dynamic routing was configured between the two routers, which successfully achieved a FULL neighbour state and automatically propagated all four VLAN subnets across buildings without any static routes. DHCP servers were configured with ip helper-address to distribute IP configuration across VLAN boundaries, and all six PCs received correct addresses automatically. DNS A records were added so devices could be reached by name rather than by IP.

All tests passed successfully — ping between buildings confirmed cross-building connectivity, inter-VLAN ping confirmed routing between student and staff networks, traceroute showed the expected 3-hop path through both routers, and DNS resolution worked correctly for both local and cross-building domain names. Minor observations such as the first ping packet timing out due to ARP resolution and the OSPF neighbour initially showing 2WAY/DROTHER state before converging to FULL were expected behaviours resolved through correct priority configuration.

Hence the project was completed with a proper knowledge and implementation of Campus Network Design using VLANs, OSPF, DHCP, DNS, and Subnetting using Cisco Packet Tracer.